For the app, don’t make it too complicated. The app should mainly be a dashboard for live workout feedback and sensor testing. Since this is a prototype, the goal is to prove that the wearable can collect data and turn it into useful lifting feedback.

## App goal

Gym WRO app = live dashboard that shows muscle activity, motion quality, rep tracking, and fatigue/near-failure feedback.

The RGB LED gives quick feedback during the set, and the app shows the deeper data.

## MVP app features

### 1. Connection screen

The app should connect to the ESP32 through either:

Bluetooth/BLE if they can do it

Wi-Fi/local web dashboard if easier

USB serial to laptop as the simplest backup

For the first prototype, I honestly think a laptop web dashboard is easier than a full phone app. The ESP32 can send data to a computer, and the dashboard can display it in real time.

### 2. Live workout dashboard

This should be the main screen.

It should show:

Rep count

Current rep speed

Range of motion

Muscle activity level

Fatigue / effort level

Rep quality status

Near-failure indicator

The main visual should be something like a gauge:

Green → Yellow → Orange → Red

Same idea as the RGB LED.

Example statuses:

Good reps

Fatigue building

Near failure

Rep quality breaking down

### 3. Live graphs

This would make the prototype look way more serious.

Add 2 simple live graphs:

Graph 1: Muscle activity over time

This comes from the EMG sensor.

Graph 2: Motion / rep speed over time

This comes from the IMU.

Optional third graph:

Rep quality score per rep

The app does not need perfect science yet. The important thing is showing that the sensors are producing meaningful trends.

### 4. Rep-by-rep summary

After a set, show a quick summary:

Total reps

Average rep speed

Last rep speed

Estimated fatigue level

Range of motion consistency

Number of “good reps”

Number of reps where form/quality started dropping

Example:

Set Summary

12 reps completed

Fatigue increased after rep 8

Rep speed dropped on reps 10–12

Range of motion decreased near the end

Final status: Near failure

### 5. Calibration / setup screen

Before the set, the app should allow a quick calibration.

Something simple:

Select exercise: Biceps Curl

Press Start Calibration

User performs 2–3 normal reps

App records baseline motion and muscle signal

Then user starts the actual set

This is important because everyone’s EMG signal and rep speed will be different.

### 6. Data logging

The app should save basic test data.

At minimum, save:

Time/date

Exercise

Rep count

EMG values

IMU values

Fatigue score

Notes

Even exporting as a CSV file would be good. That helps later if you want to analyze the data.

## What the app should NOT focus on yet

Do not waste time on:

Login/signup

User profiles

Social features

Perfect mobile design

AI coach voice

Fancy animations

Full workout planner

Those are future features. For now, the app needs to prove the prototype works.

## Best app structure

I’d tell your teammate to build it like this:

### Screen 1: Device Connection

Connect to Gym WRO wearable.

### Screen 2: Calibration

Set baseline for muscle activity and motion.

### Screen 3: Live Set

Show live rep count, effort gauge, rep quality, and graphs.

### Screen 4: Set Summary

Show what happened during the set and save/export data.

## Simple data the ESP32 could send

The ESP32 can send something like this every few milliseconds:

{
  "time": 1.25,
  "emg": 642,
  "accelX": 0.12,
  "accelY": 0.85,
  "gyroZ": 14.2,
  "repCount": 5,
  "rom": 82,
  "repSpeed": 1.4,
  "fatigueScore": 62,
  "status": "Fatigue Building"
}

The app just reads that and displays it.

## My recommendation

For the prototype, tell them to make a web dashboard first, not a full phone app.

Best stack:

Frontend: React, HTML/CSS/JavaScript, or simple web app

Data input: Bluetooth, Wi-Fi, or USB serial

Charts: simple live line charts

Export: CSV download

That is realistic and impressive enough for the Innovation Challenge.

The app should answer one question clearly:

“What is happening during this set that a normal smartwatch cannot show?”